# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of equipment under the project “Clean and green life”	p 1 /…

Publication reference : CB007.1.32.224-2

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Special vehicle for collection and disposal of waste Quantity: 1 Year of production: 2016. or later Gross vehicle weight min. 26.000kg Vehicle configuration (wheels) min. 4x2 Engine power min. 220kW - DIESEL Engine torque min. 1.300Nm Engine emission control: Min. Euro 6 Gearbox manual with min. 16 speed Adjustable steering wheel with servo – left side Safety: ABS, AEBS Advanced emergency breaking system, ESP Electronic Stability program Alarm for reverse moving Digital tachograph for two drivers Accompanying equipment: spare tire, wheel wrench, crane for mounting a wheel, triangle Air condition in cabin Cruise control Front under run protect Fog lights Electronic windows Cross axle differential lock Min. overhaul 100.000km (a warranty technical maintenance to be performed at 100.000 km) Cabin capacity min. 2 persons Superstructure characteristics: Superstructure with rear loading by the principle of an ejection plate and a bucket Max vehicle and cargo weight must be adjusted to truck’s capacity The volume of the superstructure hopper (payload) min. 18 m3 Max vehicle height 3,700 mm Max vehicle width 2,600 mm Hopper bottom made of a material resistant to wear (min. HARDOX 400 or equivalent) in a thickness of min. 8 mm Pusher plate – door min. volume 2m3 Cylinders to move a sliding panel of the mechanism for compacting located on the inner side of the pusher plate – door Pusher plate – door must automatically lock and unlock when opening and closing Self-cleaning of the door when discharging Sealing between the door and the hopper with a rubber profile which prevents any leakage Automatic/manual control option for moving of the ejection plate along the loading hopper while pressing with min. level of compression of 6: 1 The front part of the hopper deepened for collecting drain water, with capacity of min. 150 l Management of all superstructure functions, excluding the shaker, is electro-hydraulic using a programmed module Failures diagnostics by means of indication of input and output signals in the chassis cabin Single and continuous automatic cycle of pressing mechanism work. One cycle duration is max. 30 sec. Double pump for the independent work of waste compacting and containers lifting device Bins lifting device is adapted to work with garbage bins from 80 1 to 240 1 and containers of a volume of 1.1 m3. Separate mechanism for discharging of containers of volume from 5m3 to 7m3, made according to standard EN 840-1,2,3 Min. bins lifting weights 700kg All cylinders with built-in G-sliding bearings with lubricators The most loaded bolts chromed on devices for tipping containers Two collapsible pedals and handrails for workers are in the back on both rear sides of the vehicle Two working lights on the rear side of the vehicle. Other light equipment of the vehicle according to ECE R 48.02 Yellow flashing lights, one on the cab of the vehicle and the other at the rear of the superstructure Emergency button Protection and color of the superstructure (all elements are sandblasted, painted with a two-component primer, the final layer is a two-component polyurethane (PUR) paint, drying in a drying chamber at high temperatures) Color of the superstructure – orange |  |  |  |
| 2 | Mini backhoe loader Quantity:1 Year of production: 2016. or later Max. Nett engine power (acc.to ISO 14396) min. 70kW (up to 2,800 rpm) Engine emission level min. acc.to TIER IV (or equvivalent) final standard to achieve lower CO2 emission level, high productivity and highest fuel consumtion efficiency. Displacement max. 3,5 lit. Min. 4 cylinders. Hydrodinamique transmission Movement speed min. 30 km/h Rear axle differential lock Operating weight min. 8000 kg Loader – multipurpose front loading shovel min. 1m3, with forks Backhoe – excavating device integrated at the rear side of the machine. Digging depth min. 2,5m Hydraulic pump flow rate min. 150 l/min Hydraulic oil pressure min. 200 bar Working lights Rotating beacon Airconditioning device Hammer pipework (hydraulic installation)-hydraulic attachemnts possibility Pneumatics – front: min. 18’’; rear: min. 28’’ Digging bucket width min. 500mm Accompanying equipment: lubricator, wheel wrench Operating manual in Serbian languague Spare parts catalogue Service support provided in Serbia for up to 300km distance from Knjazevac Municipality, Serbia |  |  |  |
| 3 | Dump truck Quantity:1 Year of production: 2016. or later Lenght min. 6.000mm Engine min. EURO 6 Gross vehicle weight max. 3.500kg Payload of complete vehicle min.1.300kg Gearbox, manual, 6+1 gears Steering wheel with servo, left side Drive 4x2 Frames: Section shape C Brakes– min. ABS8, EBD, ASR, MSR Mechanically controlled parking brake Manual air conditioner Cab with a seat for min 2 persons Min. 4 tyres – summer, steel wheels Adjustable driver seat with safety belt Safety belts for passenger Board computer containing min. information on waring messages on fuel consumption, time, distance traveled and operation of additional devices Headlight alignment from the cab Immobilizer Remote central lock Accompanying equipment: auxiliary tool, crane for wheels, triangle, wheel wrench Heated electric adjustable mirrors Electronic windows Rear fog lights Instructions for operation and maintenance in Serbian language Colour - white Spare wheel with tyre equal to front wheel Superstructure: Min. lenght 3.500 mm, min. width 2.100 mm, min. height 350 mm Aluminium sideboards Fenders with curtains 2+2 side LED marker light – homologated |  |  |  |
|  | Waste container 1100l Quantity: 40 Volume: 1100 l Dimensions according to EN 840 3,5,6 Material: High quality deep pressed steel sheet min. thickness of 1,25mm Mechanism for setting the lid Container has two supports on the sides for supporting the discharge Two handles on the left and right sides and two handles on the font side Dome lid operating on reversed hammock springs system and with protective rubber along the front edge of the lid Two lid stops on the body sides Hot-dip galvanizing in accordance with EN ISO 1461 Four wheels with rubber coating min. hardness of 80 Schor Wheel diameter min. 200mm Load point min. 400kg per wheel Angle of rotation wheel is 360º All metal parts of wheel are galvanized Two front wheels with wheel block (foot activation) Container has a water outlets on both sides Wheel carrier with thickness min. 3mm Wheel carriers are welded from both sides All 4 container angles are marked with a fluorescent red and white stripes, dimensions 400x200mm Strengthening the bottom of the container according to minimum load point per wheel |  |  |  |